DOE Phase I SBIR/STTR Final Technical Report

FEASIBILITY OF STORING HEAT IN THE SUBSURFACE FOR FLEXIBLE ELECTRICITY GENERATION

TOPIC AREA 10: GEOTHERMAL - ADVANCED HEAT STORAGE

Business Point of Contact

Projeo Corporation

2409 Berniece Dr.

Champaign, IL 61822

Technical Point of Contact

Nick Malkewicz

Projeo Corporation

Phone: +1 217 244 0433

nwmalk@projeoco.com

Team Members

Nick Malkewicz (Projeo Corporation)

Dr. Tugce Baser (Department of Civil and Environmental Engineering, University of Illinois)

Dr Steve Whittaker (Director Energy & Minerals, Illinois State Geological Survey, University of Illinois)

Contents

Project Overview	4

Background	6

Target Reservoir Evaluation	6

Petrophysical Analysis	6

Potential Formation Analysis	11

Target Reservoir Selection	11

Preliminary Regulatory Evaluation	14

Well Site Selection and Setup	14

Well Information & Details	15

Well Conversion	17

Field Test	18

Testing Setup	18

Monitor Hot / Cold Injection & Thermal Stabilization	31

Site Closure	35

Data Acquired & Instrumentation	38

Well Logs Acquired	38

DTS Real Time	38

Downhole Memory Gauge	38

Surface Instrumentation	38

Samples Acquired	38

Field Test Results	42

Initial Temperature Profile	42

Temperature Profile From DTS	42

Time series of temperature	45

Downhole Pressure/Temperature	46

Wellhead Pressure/Temperature	47

Interpretation of Pressure Transient Testing	47

Numerical Simulations	49

Model Validation With Existing Data Set From The Literature	49

Model Calibration With Field Test	51

Initial and Boundary Conditions	54

Simulation and Analyses	55

Simulation Results	56

Power Generation Calculations	59

Phase I Market Research & Business Model	62

Economic Model	63

Team for Phase II	64

Additional Custom market Research	65

Lessons Learned	65

Conclusion	67

References	68

Appendices	69

Project Overview

The phase I research on the Feasibility Of Storing Heat In The Subsurface For Flexible Electricity Generation project focused on the characterization of subsurface thermal properties of the subsurface in Illinois to investigate the feasibility of advanced heat storage approach for electricity generation via existing wells to support grid flexibility. The major goal of this project was aligned with the Advanced Energy Storage Initiative (AESI) that focuses on research and development that improves flexibility and grid services, thereby cost-effectively improving (electric grid) reliability. Using existing oil and gas wells is appealing and can help improve the cost competitiveness of electricity generated from geothermal power plants. However, the major challenge is to determine the characteristics and suitability of the wells for thermal storage. Many wells across the USA are owned by private parties and were constructed many years ago and there is little information on their productivity or reasons why they were abandoned. Moreover, identifying the specific characteristics of wells for suitability for thermal storage needs to be investigated.

A recent report by the DrillingInfo reported around 3.6 million abandoned wells, of which 69% are unplugged. We are proposing that these wells can be utilized for advanced geothermal energy storage (AGES) systems in subsurface formations to generate electricity. This approach involves collecting heat from renewable or industrial sources at the surface and injecting it into the existing wells to create a sustainable and artificial geothermal underground reservoir. The conceptual schematic of an AGES system is shown in Figure 1. The system involves multiple wells to inject and extract heat into/from geological formations and the magnitude of the injection depends on the heat availability from the surface infrastructure, which is shown as heat exchanger in Figure 1. The heat exchanger for instance, can be replaced with a solar array system coupled with the organic Rankine cycle geothermal generator. The main objective of this approach is to store the heat and increase the temperature of the existing reservoir sufficiently so that the extraction temperatures are enough to generate electricity, or optimally for direct heating and cooling. Identifying and converting appropriate oil and gas wells to geothermal sources of clean energy, prior to abandonment, has many advantages. In addition to protecting the environment, additional revenue streams from geothermal energy can be generated for many years. Despite the potential and numerous advantages, the existing studies have focused on only hypothetical scenarios except for a few projects in France and United Kingdom. There is a need for a full-scale demonstration, using available field measurements and modeling data to investigate the feasibility of generating electricity by injecting heat into the geothermal storage reservoir.

Figure 1 – Example schematic of an advanced heat storage system

The phase I project was focused on developing a process of storing heat generated by renewable energy resources in the subsurface via existing oil and gas wells for later use for electricity generation. To this end, the Phase I field test investigated the heat storage and hydrogeological characteristics of subsurface reservoirs in the Illinois Basin. The project team performed preliminary analyses to determine the thermal properties of the subsurface using data obtained from existing wells in the Illinois Basin. The project team had access to over a decade of high-resolution Fiber Optic temperature data from another DOE-funded (FC26‐05NT42588) project, the Illinois Basin Decatur Project (IBDP). The distributed temperature data used temperature perturbations from that project and the highly dense well logs data set and geologic models already created to help classify high-potential geothermal reservoir candidates in the basin. The team performed modeling and simulation on the identified reservoir to evaluate the heat losses of returning fluids during heat extraction. The ideal reservoir response to heat injection was evaluated for determining the evolution of temperature profiles and heat losses over time and the target reservoir/well site location for the field test was identified. A small-scale field test was designed using an existing oil well to refine and calibrate the model and to begin to demonstrate the ability to navigate production company interface, permitting, and regulatory pathways necessary for the conversion of oil and gas assets towards geothermal use. The team then developed preliminary project economics and lifecycle costs for a larger scale Phase 2 demonstration.

The M. Eckleberry #4 well site is in the Illinois Basin Northwest of Fairfield, IL and the target reservoir selected was the Cypress Middle Formation (Top of formation at 2,905’ Ground Level (GL)). The test included initial sampling and in-situ pressure measurement followed by installation of Distributed Temperature Sensing Fiber Optic cable, a downhole memory gauge, and surface instrumentation to monitor a hot water injection period followed by thermal decay period, and a cold-water injection period also followed by a thermal decay period. Additionally, a step rate test and a pressure fall off test were incorporated in the cold injection to help refine the reservoir properties. The results obtained were used to calibrate the numerical models discussed in the field simulations section of this report.

# Background

The phase I project was focused on developing a process of storing heat generated by renewable energy resources in the subsurface via existing oil and gas wells to later use for electricity generation. The overall objective of this project was to provide proof of concept via field hot fluid injection test to characterize the heat storage properties of the subsurface in the Illinois Basin.

The Phase II work plan was designed to satisfy the project objectives and consisted of three steps: (1) estimation of in-situ geothermal potential (2) validation of computational tools to explain thermo-hydro-mechanical response via field experiment; and (3) economic model for thermal storage utilization. The first step focused on the characterization of geothermal reserves of a formation, the transfer of those heat reserves to a working fluid, the delivery of that fluid to the surface and the ability to convert those fluids to usable energy. The Phase I field test was intended to investigate the heat storage and hydrogeological characteristics of subsurface reservoirs in the Illinois Basin. The field test would enable the characterization and calibration of thermal properties in subsurface geomaterials to evaluate suitability for thermal energy storage. Additionally, several studies existed on the feasibility and the concepts of subsurface heat storage, but an actual field test had not been implemented. The team intended to perform a field test to provide an initial procedure for converting oil and gas wells and to establish the regulatory requirements and pathways for the conversion process. The third step dealt with developing an economic estimation model that accounts for uncertainty in the preliminary study through Monte Carlo simulation.

# Target Reservoir Evaluation

To identify a physically viable oil well in Illinois, the team first reviewed the well database of Illinois State Geological Survey (ISGS) and used existing oil and gas dataset pertaining to a DOE-funded Carbon Capture Storage project for the Illinois Basin. The distributed temperature data used temperature perturbations from that project and the highly dense well logs data set and geologic models already created to help classify high-potential geothermal reservoir candidates in the basin. The petrophysical, geological, and hydraulic data provided the much-needed information about the subsurface characteristics, i.e., lithology and mineral composition, porosity, initial temperature profile, time series temperature evolution in the subsurface and hydraulic conductivity (permeability) were used to evaluate the thermo-hydraulic properties of the geological formations. These datasets were combined to perform a high-level screening for the target formation for heat injection. Priority was given to formations which were prevalent throughout the basin and penetrated by most oil wells in throughout the Illinois Basin.

The team intended, to pick a reservoir that was prevalent across the Illinois basin and most oil and gas wells would likely penetrate a similar formation, for the purpose of profiling a potential reservoir for a commercial project. The geologic expertise at the ISGS Oil and Gas (O&G) industry professionals at the site host Podolsky Oil Company were consulted during the process of selecting the target reservoir.

### Petrophysical Analysis

Petrophysical interpretation of a formation can provide information on the assessment of properties of the reservoir such as porosity, fluid saturations, permeability, lithology, and mechanical properties of geological profile. These results can provide significant information for determining the optimal layer for a geothermal storage reservoir. Gamma ray (GR) logging is used to characterize the rock type in a borehole by measuring gamma radiation. High gamma rays are indicative of shale formation whereas lower values are more typical for limestone and sandstone. Different types of rock emit different amounts and different spectra of natural gamma radiation; therefore, formation types can be identified based on the gamma ray logs as shown in Figure 2. Although the geological profile in the Illinois Basin consist of geological formations deposited in different times, here they are represented in groups based on rock types (i.e., shale, limestone, sandstone, etc.) for simplicity.

Figure 2 - Gamma log from CCS #1 well

The resistivity of a geomaterial is a fundamental material property, which represents how strongly a material opposes a flow of electric current. Low resistivity indicates that the formation is highly porous and rich in salty water. On the other hand, high resistivity can be linked to low porosity and presence of hydrocarbons. The resistivity logs from CCS #1 well are given in Figure 3.

Figure 3 - Apparent resistivity profiles from CCS #1 well

The thermal and hydraulic properties of a geomaterial depends on porosity. For high porosity values for a given geomaterial, the higher the porosity the lower the thermal conductivity because of the higher volume of water in a geomaterial. However, higher values of thermal conductivity are not desirable for a heat storage system because of the correlation with higher heat loss and a resultant reduction in efficiency. Therefore, a desirable porosity value of an advanced heat storage system is a combination of thermal and hydraulic properties of the geomaterial. Total porosity is defined as the fraction of pore volume in a volume of rock meaning it includes isolated pores while effective porosity represents the volume of the connected pores in a volume of rock. The total and effective porosity profiles are shown in (Figure 4), respectively. These values are used as material properties in the numerical modeling efforts.

Figure 4 - Porosity values for different formations Total porosity (Left) Effective porosity (Right)

Figure 5 - PERMEABILITY PROFILES FROM CCS #1 WELL

The permeability profile from the CCS #1 well is presented in Figure 5. High permeability is expected in sandstones and lower values are estimated to be in the shale and siltstone layers. In shale and siltstones, particle sizes are smaller; this means they have larger specific area, which will decrease in turn the size of the pore channels leading to lower permeability values. Thermal conductivity and diffusivity are important thermophysical rock properties, needed for heat flow determination, deep thermal regime assessment, and reconstruction of thermal history of sedimentary basins. In contrast to temperature logging, in-situ measurements of thermal conductivity in wells are time consuming and technically difficult. Thus, thermal conductivity is generally measured in the laboratory on drill cores and cuttings. If rock samples are not available, it can be estimated either from other physical properties measured in the borehole or from the mineralogical composition. In this study, we calculated the thermal conductivity values of the geomaterials from the mineralogical contents based on the volume fractions of minerals obtained from the analyses as shown in Figure 6.

Figure 6 - Volume fractions of minerals in each formation

The values of thermal conductivity from literature for the different minerals encountered in the logs (dolomite, calcite, chlorite, quartz, feldspar, illite, kaolinite, pyrite, and anhydrite) are summarized in Table 1.

Table 1- Thermal conductivity of different minerals

| Minerals | Thermal conductivity | Minerals | Thermal conductivity |
| --- | --- | --- | --- |
|  | (W/mK) |  | (W/mK) |
| Anhydrite | 4.76 | Illite | 2 |
| Calcite | 3.57 | Feldspar | 2 |
| Chlorite | 5.14 | Kaolinite | 2.8 |
| Dolomite | 5.5 | Pyrite | 19.2 |
| Quartz | 8.4 |  |  |

Using the mineral fractions along with their corresponding thermal conductivity, the formation thermal conductivity can be mixture can be calculated using Equation 1 (Farouki et al, 1981) as follows:

| λs=jmλjxj | (1) |
| --- | --- |

where ∏ is the product of λ factor and x are the volume fraction of the mineral. The results from the calculations are presented in Figure 7.

Figure 7 - Calculated thermal conductivity values

### Potential Formation Analysis

Initial analyses before the field test were performed using the existing data pertaining to the petrophysics, thermal and hydraulic properties of geomaterials, in addition to seismic logs. Downgrading of formations was further performed according to the analyses, and the physically viable targets were identified as shown in Table 2.

Table 2 - Formations selected for further investigation.

| Formation | Lithology | Depth | Thickness | Total Porosity | Permeability | Thermal Conductivity |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (m) | (m) | (m3/m3) | (m2) | (W/m•K) |
| Golconda | Limestone | 644 | 87 | 0.16 | 6.65x10-15 | 3.45 |
| Menard | Limestone | 731 | 38 | 0.03 | 1.26x10-17 | 3.57 |
| Vienna | Limestone | 769 | 36 | 0.06 | 34.04x10-16 | 3.35 |
| Tar Springs | Sandstone | 810 | 53 | 0.15 | 1.22x10-15 | 3.17 |
| Cypress | Sandstone | 868 | 45 | 0.18 | 6.02x10-17 | 3.50 |
| Benoist | Sandstone | 921 | 11 | 0.03 | 1.15x10-17 | 3.62 |
| Renault | Sandstone | 932 | 24 | 0.03 | 2.83x10-17 | 3.64 |

### Target Reservoir Selection

Based on the conducted analysis and consultations with the geologic expertise at the ISGS and O&G industry professionals at the site host Podolsky Oil Company the Cypress Middle Formation (Top of formation at 2,905’) was selected for the field test. The Cypress Sandstone is prevalent across the Southern Illinois Basin and extends into Indiana and Kentucky (Figure 8). The Illinois stratigraphic column highlighting the Cypress Sandstone is presented in Figure 9.

Figure 8 –  Regional Extent of the Cypress Sandstone in the Midwest (Richard C. Berg n.d.).

Figure 9 –  Illinois Basin StratiGraphic COlumn (Richard C. Berg n.d.).

# Preliminary Regulatory Evaluation

Understanding the regulatory pathway for converting oil and gas wells for an advanced geothermal energy storage, were initiated by interviewing the regulatory bodies in the state of Illinois, to understand the regulatory environment for a commercial AGES project. Projeo conducted a preliminary review of existing regulatory frameworks and identified that the Illinois Environmental Protection Agency (IEPA) would be responsible for oversight of the AGES system. However, after preliminary discussions with IEPA in October 2020, it was further identified that any permit conversion would need to be done in collaboration with the Illinois Department of Natural Resources (IDNR) which is the regulatory body overseeing the existing O&G permits. On October 22, 2020, an additional meeting with representatives from INDR confirmed that IEPA would have jurisdiction over the AGES wells and that IDNR did not have a process to transfer an existing O&G well from IDNR to IEPA (agency to agency). The Interim Director of the IDNR, mentioned that this was a novel approach in the state of Illinois. Nevertheless, he voiced that the agency would be willing to discuss and evaluate a pathway for transferring the O&G well permitted under IDNR to IEPA, for use in AGES system and that the burden of meeting and proving that the well(s) met IEPA regulatory requirements would be on the operator. Nonetheless, interagency collaboration would be critical. A follow-on meeting with both agencies took place on November 17, 2020. During this meeting, the agencies agreed that interagency collaboration would be necessary and that there may be precedent with some United States Environmental Protection Agency projects which could be leveraged. The discussions were focused on the regulatory requirements associated with the wells and the process of transferring the wells under IDNR to IEPA classifications. The consensus was that a project would need to be identified and the operators would need to work with both regulatory bodies to allow for conversion. Both agencies expressed a willingness to work through the process once a project was identified. Projeo noted that upon identification a candidate project additional discussion would begin. Both agencies IEPA and IDNR left the meeting with actions to begin looking at what internal processes and procedures would be necessary to undertake the conversion. Overall, the preliminary steps for identifying a regulatory pathway for an AGES project in Illinois were successful in that the regulatory bodies expressed a willingness to determine a regulatory path forward to develop such a project once a candidate site was selected.

# Well Site Selection and Setup

Projeo Corporation reviewed the initial analysis of target formations with the site host partner Podolsky Oil Company, to identify a candidate well site for the field test. The M. Eckleberry #4 site was proposed. After assuring appropriate site geologic conditions, the team conducted a site survey of the M. Eckleberry #4, on December 7, 2020, to evaluate the site surface infrastructure for suitability. The well site provided by the site host was planned to be plugged and abandoned (P&A). Therefore, Podolsky Oil Company, benefitted by providing the well for the field test and as a result, reducing their costs associated with P&A operations. The site survey involved evaluating road conditions for access to well site, power, and water availability at well site, and site layout for operational activities. In addition to the site survey well conditions (well design, completion history, mechanical integrity, and associated geologic formation and aquifer characteristics) were also evaluated. The M. Eckleberry #4 well was selected by Projeo Corporation for the field test, due to the geologic setting, age of the well, completion history, and availability of offset well log data. The field-testing operations were delayed due to COVID-19 and then winter weather conditions and Illinois road restrictions. The site setup operations were initiated on March 10, 2021, after the frost laws were lifted. Site setup activity included improving site road accessibility, spotting a frac tank, office trailer etc. The well layout is depicted in Figure 10 and the detailed site activity is presented in Appendix A: Field Activity Report.

Figure 10 – Site Layout

### Well Information & Details

The M. Eckleberry #4 well in the Illinois Basin, was drilled from August 19, 2013, to August 22, 2013, and is located 480' North and 660' West of the Southeast Corner Northeast Quarter of the Northeast Quarter of Section 16, Township 01S Range 07E of Wayne County, IL (Lat 38.440375 Lon -88. 424731) (Figure 11). The M. Eckleberry #4 as-built construction parameters are presented in the well layout as depicted in Figure 12. The well ground level elevation was 446’ and the kelly bushing depth was 451’. All depths referenced in this report are from the kelly bushing depth.

Figure 11 – M. Eckleberry Well Location in the Southern Illinoi Basin.

Table 3 – WellBore Construction Details

| Section | Drilled Depth (feet) | Casing Depth (feet) | Bit Size (in) | Casing Size (in) | Other Information |
| --- | --- | --- | --- | --- | --- |
| Surface | 263’ | 253’ | 12-1/4” | 8-5/8” | Cemented to surface |
| Intermediate | 3,146’ | 3,126’ | 7-7/8” | 4-1/2” | Top of Cement at 1,300’ Lead: 200 sks of 65/35 Poz-A blend w/6% gel and FLA Tail: 150 sks of 50/50 Poz-A blend w/ 3% KCL and 2% Calseal |
| Production | 3,146’ | Open Hole | 7-7/8” | N/A | Aux Vases formation, stimulated with 500 gal 15% HCl, and fracked with 4800 gal 15# Crosslink, 7500# 20/40. |

Figure 12 – M. Eckleberry #4 Well LAYOUT.

### Well Conversion

The M. Eckleberry #4 well was drilled and completed as an oil producing well. A procedure to test and convert this oil producing well to an Advance Geothermal Energy Storage (AGES) test well was developed by the Projeo Corporation. The procedure developed would serve as a starting point to evaluate and convert orphaned and abandoned oil and gas wells into a viable AGES well. The field program is attached in Appendix B: Field Program.

On March 22, 2021, a wireline gauge ring was run to check casing integrity for obstructions in the well was conducted prior to isolating the Aux Vases open hole production zone. The production zone of the well was then isolated with a Cast Iron Bridge Plug (CIBP) and cement was bailed on top of the CIBP creating a 10-foot cement column in the well per IDNR regulations.  Once the production zone was isolated, a cement bond log (CBL) was acquired to ensure the casing and cement integrity, a casing collar locator log was acquired to verify completion tally and a gamma ray log to was acquired to correlate formation tops with offset well log data. Additionally, a mechanical integrity test was performed on the casing and the CIBP by pressurizing the well to 500 psi at surface (Figure 13).

Figure 13 – Casing Pressure test Performed on March 22, 2021.

Figure 14 – CEMENT bond log, casing collar locator log and Gamma ray log tool ASSEMBLED ON left and cement bailer on right.

# Field Test

### Testing Setup

The gamma ray (GR) log acquired was evaluated by the team and compared to the offset well data to refine the perforation interval of the Cypress Formation (Target Reservoir). Based on the available data a 5-foot interval from 2,958’ to 2,963’ (901.5m and 903m) was selected for perforation. On March 23, 2021, the well was casing swabbed with a swab rig (Figure 16) to allow for underbalanced perforating. The underbalanced perforation helps minimize perforation skin damage and helped to improve formation cleanup for sampling. Additionally, it enabled initial estimates of in-situ pressure and permeability through monitoring the formation water flow and pressure buildup at surface. The well head was configured with adequate pressure control equipment to manage fluid influx (Figure 17). The perforating guns were loaded with 21 grams of deep penetrating titan charges 4 shots per foot with 90 deg phasing for 5 feet (0.43” entry hole/42.13” penetration), as per API specification for the well casing (Figure 17). The well was shut in for 30 minutes following the perforation and surface pressure was monitored using Dwyer PR2000 data logger. After monitoring the well pressure buildup and pressure stabilization, the well was casing swabbed (Figure 19) to purge the near well bore formation fluids and acquire native formation fluid samples. The fluid level was initially tagged at 219’ before the first swab pull, and a total of 92 bbls were purged before acquiring a 5-gal formation fluid swab sample (field density was 8.8ppg). Two additional samples were acquired with the Kuster Flow Through Samplers (Figure 18) using the rig sand line at a depth of 2,900’ after purging approximately 136 bbls of formation fluid. The Kuster Flow Through Samples is timed with a clock to close a flow through valve (Figure 18), which assists with acquiring a well sample at a specific depth in the well. Detailed lab analysis results of acquired samples are discussed in the “Data Acquired & Instrumentation” section of this report.

Figure 15 – Pressure BUILDUP MONITORED AFER PERFORATING THE CYPRESS FORMATION, ON MARCH 23, 2021.

Figure 16 – Swab Rig as photogrpahed on March 23, 2021.

Figure 17 – Well head configured with Orbit VALVE, PUMP – in SUBS (Flow -Tee) and LUBRICATOR (on Left), Perforation Gun after charges fired (right).

Figure 18 – Kuster sampler assembly on rig sand line (left), Clocks used to time the sampling (Right)

On March 24, 2021, the static fluid level was tagged at 218' with the swab rig and the cement on the CIBP was tagged at 3,095' indicating minimal or no sanding (sand flow from the formation into the well). A PPS 58 (S/N 90921) memory gauge (Figure 20) was deployed in the well to record pressures (Figure 21). The gauge was held stationary for gradient stops for 2 minutes every 100’ from 0’-500’, and 2 minutes every 500’ from 500’-2,500’. The gauge was then held stationary at 2,958’ (top perforation) for 15 minutes. In-situ Cypress Pressure was 1,291 psia and temperature was approximately 97F. The gauge then softly tagged bottom at 3,096’ and was held stationary for 2 minutes at the tag depth. The gauge was also held stationary for pressure gradient stops for 2 minutes at 3,000’, 2,958’, 2,000’, 1,000’, 225’, 210’, 200’ on the way out.

Figure 19 – Casing Swab SETUP TO Acuire a swab sample (Left), PPS 58 (S/N 90921) memory gauge deployed to record well pressures (Right).

Figure 20 –PPS 58 (S/N 90921) memory gauge as Photographed on March 24, 2021.

Figure 21 – WELL Pressure and temperatures recorded by the PPS 58 (S/N 90921) memory gauge.

On April 1, 2021, the DTS fiber optic cable was deployed in the well, the cable specifications and data sampling is summarized in the Data Acquisition & Instrumentation section of this report. The DTS spool was mounted on a winch truck (Figure 22) and lowered in to the well through a sheave wheel hoisted by a crane (

Figure 23 - DTS CABLE SETUP FOR INSTALLATION THROUGH THE SHEAVE WHEEL AND CRANE.

). The downhole assembly of the DTS cable consisted of a DTS cable housing assembly a weight bar and a pressure/temperature memory gauge (PPS 58/ S. No# 90921) (Figure 24). A dual hydraulic Pack off was installed on the well head for pressure control and the DTS cable was secured with a T-bar (Figure 25). Once the DTS was securely installed in the well it was connected to the Silixa DTS interrogator and the site computer. A heat test was applied on a 1’ interval above the DTS clamp to confirm the length of the DTS cable in the well and on surface. The bottom of the DTS cable was installed at 3,091.5’/942.3m KB, and 254.71’/77.63m of DTS cable was on the surface above KB depth. Based on the DTS cable length, the perforation interval of the Cypress formation on the DTS cable was at 3,207.71’ /977.7m to 3,212.71’/979.2 and the bottom of the PPS 58 memory gauge was at 3095.5’/943.5m KB. The downhole assembly is summarized in Table 4 below.

Table 4 – tally of the downhole assembly

| Downhole Tally |  |  |
| --- | --- | --- |
| Component Description | Component Length (inches) | Cumulative Length (inches) |
| Memory Gauge | 15.25 | 15.25 |
| Battery Housing | 8.125 | 23.375 |
| Cross-Over | 3.25 | 26.625 |
| Cross-Over | 3.125 | 29.75 |
| Weight Bar | 60 | 89.75 |
| Cross Over | 5.5 | 95.25 |
| DTS Splice Housing - 1 | 19.375 | 114.625 |
| DTS Splice Housing - 2 | 5.75 | 120.375 |
| Cross-Over | 1.375 | 121.75 |

Figure 22 – DTS Cable Spool Mounted on the WInch Truck.

Figure 23 - DTS CABLE SETUP FOR INSTALLATION THROUGH THE SHEAVE WHEEL AND CRANE.

Figure 24 – Downhole Assebly with DTS Cable Housing, Weight Bar, and Memorry Gauge.

Figure 25 – DTS cable installed and secured with DUal Pack-off and T-Bar.

On April 1, 2021, the surface flow lines (Figure 26), heater and pumps (Figure 27), and the flow line instrumentations were setup for hot/cold water injection and thermal decay monitoring. The surface instrumentation is further described in the “Data Acquired & Instrumentation” section of this report. All instrumentation were powered by generators and battery banks were setup as contingencies to avoid any data losses due to power loss or fluctuations. The site computer was configured for remote access using a wireless hotspot and to continuously upload the data on a secure cloud location for the team to access and assure offsite backup. Once the setup was complete, 660 gallons of fresh water was circulated through the surface system to test the instrumentation.

Figure 26 – Flow Lines and well head insulated and surface instrumentation installed.

Figure 27 – Line Heater and single pump setup for hot water injection.

### Monitor Hot / Cold Injection & Thermal Stabilization

On April 5, 2021, an auto shut in switch (Figure 28) was installed on the flow lines to shut off the pumps at high- and low-pressure thresholds, and the hot water injection was initiated at 8 gal/ min at a temperature of 50oC or 122oF. Due to skin build up at the perforations the pump had stopped injecting hot water around 8 pm after injecting approximately 19 bbl into the Cypress formation (Figure 29). Higher injection pressures were required to maintain a constant rate of injection. Hence on April 6, 2021, two high pressure triplex CAT pumps were installed on the flow line (Figure 30) to overcome the formation skin for the hot water injection. The pump specifications are summarized in Figure 31 and Figure 32 . The perforations breakthrough was observed at 796 psi and the injection pressure thresholds were adjusted to 800 psi (High)/100 psi(low) (Figure 33).

The surface instrumentation and the data acquisition system were setup to access and monitor operations remotely. Daily checks were performed by the team or site host personnel (Podolsky Oil Company) on the equipment functionality, fluid levels and general site condition.

Figure 28 – AUTOmated Murphy switch Installed to shut the pump at high or low thresholds.

Figure 29 – First Days of Hot Water injectioN stopped due to High pressures.

Figure 30 - Two additional CAT pumps installed for higer Pressures.

Figure 31 – CAT 230 Pump SPECIFICATIONS (CAT PUMPS)

Figure 32 – CAT 740 Pump SPECIFICATIONS (Cat Pumps)

Figure 33 – Hot Water injection with Additional Pumps

After 3 days of hot water injection, on April 9, 2021. the team reviewed the heat losses in the well and decided to shut in the well to monitor pressure fall off, and formation thermal stability. Approximately 417 bbl of fresh water had been injected into the Cypress formation when the well was shut in and pressure fall off was monitored for 5 days.

On April 15, 2021, a cycle of cold-water injection was initiated. The cold-water injection was conducted at an initial rate of 2 gpm for 3 hours, then it was ramped up to 4 gpm for an additional 3 hours. The injection rate was then increased to 6 gpm for additional 121 hours (Figure 34). After the cold-water injection at varying rates the cold-water injection was stopped, and pressure fall off and formation thermal stabilization were monitored. An additional 124 bbl of fresh water was injected into the Cypress formation. The formation thermal stabilization was monitored for additional 18 days.

Figure 34 – COld Water Injection with a Step Rate Test and Pressure Fall OFF Incororated.

### Site Closure

On May 5, 2021, the DTS was retrieved from the well and surface instrumentations and equipment was rigged down. The well was then plugged and abandoned (P&A) on July 9th, 2021, and the P&A operations were witnessed by an IDNR inspector. The P&A operations involved setting a CIBP at 2,900’, with 2 sacks of cement on top of it. Additionally, a surface cement was set from 300’ to surface after retrieving the 4.5” casing from surface to 1,213’.  The cement truck and rig setup at the field are illustrated in Figure 35 and Figure 36.

Figure 35 – Cement truck for P&A Operations.

Figure 36 – Rig & Cement Truck Setup for P&A Operations.

# Data Acquired & Instrumentation

### Well Logs Acquired

Three logging tools Gamma Ray (GR), Cement Bond Log (CBL) and Casing Collar Locator (CCL) were deployed in the well. The CBL, tool was deployed to examine the condition of the cement bond between the well casing and the formation rocks. A good cement bond between the formation rocks and the well casing ensures that the injection fluid is isolated from flowing into unintended geologic formations. The GR log was used to categorize the formation properties at the well and correlate with available offset well data to refine a suitable injection interval in the target formation (Cypress Formation). The CCL log was used to confirm the tool depths with previously installed casing tally and ensure the other tool measurements are accurate.

### DTS Real Time

For the installation of the DTS cable, the fiber optic cable was spliced and secured in the downhole housing assembly. The (XT DTS Ruggedized) interrogator was set up and Silixa software was installed on the site computer.

The DTS fiber optic cable installed was a traditional downhole cable (0.049 wall Bare 2.50mm fibers with High Temp Acrylate). The interrogator was capable of 25 cm spatial readings at and 5 sec intervals. The sampling frequency was however set to 0.25-foot spatial readings at every minute.

### Downhole Memory Gauge

A pressure/temperature memory gauge (PPS 58/ SN# 90921) was added to the downhole assembly. The data acquired was used to refine the reservoir permeability properties through pressure transient analysis.

### Surface Instrumentation

Three temperature sensors (RTD 100/ 0 deg F to 262 deg F) pre-heater, heater bath, and post heater were installed in the flow line to assess the heat losses at surfaces. In addition, two pressure gauges, a wireless Emerson pressure gauge was placed, and a Siemens (Sitrans P/0-1600 psi) were positioned next to the well head. A wireless Emerson gauge was also installed on the frac tank to monitor fluid supply level. An Omega FTB-1425 flow meter was installed in the flow line, the flowmeter (Serial No# 0Z1908A136) has a K-factor of 885.998, and a flow range 5-50 gal/min. The RTD temperature sensors, well head pressure gauge and flowmeter sensors were connected to the Campbell Scientific Data Acquisition (DAQ) system and the data logger software was configured on the site computer.

### Samples Acquired

A total of 4 fluid samples were collected, as summarized below. An initial analysis was conducted by the Illinois State Geological Survey (ISGS) to test for Ph, density, viscosity, total dissolved solids (TDS) etc., summarized in Table 5. Additionally, a detailed chemical analysis was performed by the Illinois State Water Survey (ISWS), summarized in Table 6 - Table 7.

Details of samples acquired:

  - Swab Sample 1 – Cypress Formation

    - Time - 15:15 (~5 gallons)

    - Type - Surface Swab Line

    - Swabbed Volume Prior to Sampling – 92 bbl

  - Kuster Sampler 1 – Cypress Formation

    - Time – 16:06 (clock fired)

    - Depth/Type – Kuster 2,900’

    - Swabbed Volume Prior to Sampling – 136 bbl

  - Kuster Sampler 2 – Cypress Formation

    - Time - 16:57

    - Depth/Type – Kuster 2,900’

    - Swabbed Volume Prior to Sampling – 136 bbl

  - Injection Water Sample – Fresh Water from the Village of Geff.

    - Date – 03/30/2021 (~5 gallons)

Table 5 – Fluid Sample Initial results.

| Date | Sample ID | Temp. (°C) | pH | EC (mS/cm) | EH (mV) | DO (mg/L) | Density (g/cc) | Viscosity (cp) | Estimated TDS* (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3/25/2021 | Kuster Sample 1 | 24.4 | 5.73 | 139.3 | -389 | NA | 1.0737 | 1.3 @18.8⁰C | 110,287 |
| 3/25/2021 | Kuster Sample 2 | 24.2 | 6.31 | 129.6 | -387 | NA | 1.074 | 1.2 @19.5⁰C | 110,721 |
| 3/25/2021 | Swab Sample | 23.6 | 6.45 | 127.8 | NA | NA | 1.0742 | 1.3 @18.7⁰C | 111,011 |
| 03/30/2021 | Injection Fluid Samples | 22.2 | 8.38 | 2.515 | 397 | 7.01 | 0.9996 | 0.8 @24.3⁰C | __ |

Table 6 – Detailed Chemical Analysis of Fluid Samples, all measurementsw are in mg/L.

| Chemical Property Measured | Kuster Sample 1 (Cypress Formation) | Kuster Sample 2 (Cypress Formation) | Swab Sample (Cypress Formation) | Injection Fluid Sample (Fesh Water) |
| --- | --- | --- | --- | --- |
| Al | <0.37 ‡ | <0.37 ‡ | <0.37 ‡ | <0.37 ‡ |
| As | <1.1 ‡ | <1.1 ‡ | <1.1 ‡ | <1.1 ‡ |
| B | 2.65 | 2.55 | 2.46 | 3.17 |
| Ba | 0.276 | 0.286 | 0.273 | 0.066 |
| Be | <0.0055 ‡ | <0.0055 ‡ | <0.0055 ‡ | <0.0055 ‡ |
| Ca | 2809 | 2783 | 2832 | 78.0 |
| Cd | <0.12 ‡ | <0.12 ‡ | <0.12 ‡ | <0.12 ‡ |
| Co | <0.13 ‡ | <0.13 ‡ | <0.13 ‡ | <0.13 ‡ |
| Cr | <0.058 ‡ | <0.058 ‡ | <0.058 ‡ | <0.058 ‡ |
| Cu | 4.95 | 1.85 | <0.016 ‡ | <0.016 ‡ |
| Fe | 14.6 | 15.3 | 10.8 | 0.58 |
| K | 131 | 132 | 131 | 66.6 |
| Li | 3.8 | 3.8 | 3.8 | <1.1 ‡ |
| Mg | 1095 | 1085 | 1103 | 43.4 |
| Mn | 1.67 | 1.36 | 0.843 | 0.121 |
| Mo | <0.22 ‡ | <0.22 ‡ | <0.22 ‡ | <0.22 ‡ |
| Na | 38552 | 38549 | 38335 | 1712 |
| Ni | 14.5 | 12.8 | <0.43 ‡ | <0.43 ‡ |
| P | <0.73 ‡ | <0.73 ‡ | <0.73 ‡ | <0.73 ‡ |

Table 7 – Detailed Chemical Analysis of Fluid Samples Continuation, all measurementsw are in mg/L.

| Pb | 0.43 | 0.53 | 0.44 | <0.41 ‡ |
| --- | --- | --- | --- | --- |
| S | 408 | 405 | 410 | 76.0 |
| Sb | <0.59 ‡ | <0.59 ‡ | <0.59 ‡ | <0.59 ‡ |
| Se | <1.3 ‡ | <1.3 ‡ | <1.3 ‡ | <1.3 ‡ |
| Si | 5.1 | 5.6 | 4.4 | 6.8 |
| Sn | <0.86 ‡ | <0.86 ‡ | <0.86 ‡ | <0.86 ‡ |
| Sr | 105 | 101 | 99.7 | 5.06 |
| Ti | <0.0056 ‡ | <0.0056 ‡ | <0.0056 ‡ | <0.0056 ‡ |
| Tl | <0.47 ‡ | <0.47 ‡ | <0.47 ‡ | <0.47 ‡ |
| V | <0.47 ‡ | <0.47 ‡ | <0.47 ‡ | <0.47 ‡ |
| Zn | 1.31 | 0.992 | 0.158 | <0.097 ‡ |
| TDS | 109990 | 109500 | 110260 | 4810 |
| F | <7 ‡ | <7 ‡ | <7 ‡ | 3.1 |
| Cl | 66845 | 65952 | 65801 | 2602 |
| NO3-N | <2 ‡ | <2 ‡ | <2 ‡ | <0.06 ‡ |
| SO4 | 1049 | 1051 | 1045 | 230 |
| Br | 85 | 84 | 84 | 11.0 |
| pH | 6.76 | 6.73 | 6.81 | 8.06 |
| Alkalinity | 158 | 150 | 129 | 258 |

Data Submission

All data files were compiled to integrate and archive all project related data for future projects and to upload to the DOE OpenEI Geothermal Data Repository (GDR). The final data submissions consisted of:

- Well Logs: CBL-CCL-GR

- Surface Pressure Data: Dwyer PR2000 data logger

  - Casing Pressure Test.

  - Flow data after perforating the Cypress Formation

  - Swabbing for formation fluid samples.

- Fluid Sample Analysis Data

- Surface Data: (Campbell Scientific & Emerson)

- Downhole Data: (DTS & Memory Gauge PPS 58 90921)

- Well schematic Data.

- Numerical Analysis Data

# Field Test Results

### Initial Temperature Profile

Prior to the start of the heat injection test, the temperature in the well was recorded using Distributed Temperature Sensing system. From these recordings, the geothermal gradient at the well site was calculated. The initial temperature profile and the geothermal gradient are given in Figure 37 respectively.

Figure 37 - Initial DTS data prior to the beginning of fluid injection tests (a) Initial temperature profile (b) Geothermal gradient

### Temperature Profile From DTS

The hot fluid injection test was run for 3 days from April 5-8, 2021. The temperatures along the depth until the bottom of the wellbore was recorded using the DTS system. The evolution of temperature profiles during heating is given in Figure 38. As observed on the figures, the temperature increases on the surface to 55oC, equivalent to the temperature of the hot water being injected. The formations up to 500 m gained heat because the temperature recorded is higher than the initial temperature profile. After 500 m, the temperature recorded becomes lower than the initial profile because the heat is dissipated into the upper layers.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 38 - Temperature profiles during hot water injection (a) 2021/04/05 (b) 2021/04/06 (c) 2021/04/07 (d) 2021/04/08

After the heat injection, the well was shut in for the monitoring of thermal decay from April 9-14, 2021. The changes in temperature profiles are given in Figure 39. As can be seen in Figure 39, the temperatures decreased on the surface from 55oC to 28oC at the beginning of the shut off period, followed by a further decrease to 16oC. This temperature is still higher than the surface initial temperature recorded (~14oC).

| (a) | (b) |
| --- | --- |

Figure 39 - Thermal decay monitoring after hot water injection test (a) 2021/04/09 (b) 2021/04/14

After the subsurface temperatures reached an equilibrium, a cold fluid injection test was performed by injecting water having an average temperature of approximately 62oC between April 15-16, 2021. The temperature profiles during cold fluid injection are shown in Figure 40. It can be observed that the temperature at the bottom of the well decreased from 36oC to 29oC at the end of the cooling experiment. Due to heat losses during hot water injection, the deeper formations were displayed a higher change in formation temperature during the cold water injection

| (a) | (b) |
| --- | --- |

Figure 40 - Temperature profiles during cold water injection test (a) 2021/04/15 (a) 2021/04/16

Following the cold-water injection test, thermal stabilization and pressure fall off were monitored. The evolution of temperatures along the depth are given in Figure 41 for the period between April 17-21. Temperature at the target formation increased from 29oC to 35oC over the five-day thermal decay period.

| (a) | (b) |
| --- | --- |

Figure 41 - Thermal decay monitoring after cold water injection test (a) 2021/04/17 (b) 2021/04/21

Time series of temperature

The time series of temperatures during the field experiment for different depths are shown in Figure 42. The different thermal regimes can be identified as follows: 1) hot water injection, 2) thermal decay and pressure fall off, 3) cold water injection, and 4) thermal stabilization and pressure fall off. The temperatures increased until the hot fluid reached a depth of ~500 m, then cooling was observed in the deeper formations. This phenomenon occurred because the fluid injected lost heat to the formations in transition at relatively low injection rate. Hence, the injected hot water lost heat before reaching the target formation and the target formation fluid was not heated above 50°C. The injection rates chosen were designed by considering the geomechanical properties of the target geologic formation (to not fracture the reservoir rock). Nonetheless, the field experiments generated enough data for further study.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 42 - Time series of temperatures for different depths (a) 300m (b) 500m (c) 700m (d) 900m

### Downhole Pressure/Temperature

Pressure and temperature data were also recorded using the downhole pressure and temperature memory gauge. The data is extracted and shown in Figure 43 and Figure 44. This data was integrated into the simulations and models to refine the AGES model. Additionally, the downhole data was also used to perform pressure transient analysis to refine formation permeability estimates, discussed further in the “Interpretation Of Pressure Transient Testing” section of the report.

Figure 43 - PPS Downhole Gauge Data

Figure 44 - PPS Downhole Gauge Data.

### Wellhead Pressure/Temperature

The instrumentation was setup to record and provide real time well head pressure and temperature data throughout the field tests. This data was monitored to assess the well and the formations conditions during the hot and cold injections and for well control. Additionally, the initial formation pressures, and fluid buildup were monitored immediately after perforating the target formation (Cypress formation) at the wellhead. The buildup indicated the injectivity of the formation, without further stimulation.

### Interpretation of Pressure Transient Testing

The pressure data acquired during the field test were used to conduct pressure transient analysis and refine the target reservoir (Cypress Formation) permeability properties. The hot water injection period served as a long-term injection test, followed by a pressure fall off test during the thermal decay period. A step rate test was incorporated into the cold-water injection schedule and another pressure fall off was conducted during the thermal stabilization period after the cold-water injection.

The two fall off tests were analyzed (Figure 45) by Dr. Scott Frailey at the ISGS for an estimation of well bore storage, perforation skin, and the formation permeability. A large negative slope in the derivative of the pressure changes during wellbore storage to radial flow transition indicated a large positive skin due to partial penetration, i.e., 5 ft perforated interval of a ~53 ft thick formation. A conservative estimate of the permeability from the analysis was 820 millidarcy (8.093x10-13 m2).

Figure 45 – Type curve analysis of the two Pressure Fall Off tests (Performed by ISGS).

Based on the data and observations, a conservative formation permeability estimate was applied on the thermal storage modeling and simulations, the thermal and hydraulic parameters used in the simulation are summarized in Table 9, discussed in the “Numerical Simulations” section of the report.

# Numerical Simulations

Simulation of coupled heat transfer and water flow in geomaterials requires solving a set of governing equations for fluid flow coupled with energy conservation. The governing equations for heat transfer and water flow at the continuum scale along with other relevant equations are given in Equations 2 through 7. When formulating some assumptions are made: (a) geomaterials is homogeneous, isotropic, and non-deformable; (b) fluid flow is laminar. Darcy law is used to represent the fluid flow in the porous media using Equations (2) and (3) as follows:

| ∂∂tnrρr+∇.ρru=0 | (2) |
| --- | --- |
| u=-κrμ(∇pf+ρfg) | (3) |

where nr is the rock matrix, ρr is the rock matrix density (kg/m3), u is Darcy’s velocity (m/s), κr is the rock matrix intrinsic permeability (m2), μ is the fluid dynamic viscosity (Pa.s), pf is the fluid’s pressure in the pore (Pa), and ρf is the fluid density (kg/m3). The heat transfer in geomaterials containing fluid in the pores are governed by conduction and convection. The heat equation is derived from the first law of thermodynamics, and it is given in Equation 4. Fourier’s law was used to compute the conductive heat transfer. The convective heat transfer is caused by the density driven fluid flow because of increase in temperature in the pore fluid. Based on the energy balance in the domain, the heat transfer in the porous matrix is governed by equations (4) through (7) as follows:

| (ρCp)eff∂T∂t+ρCpu.∇T+∇.q=0 | (4) |
| --- | --- |
| q=-keff∇T | (5) |
| (ρCp)eff=1-nrρrCpr+nrρwCpw | (6) |
| keff=1-nrkr+nrkw | (7) |

where (ρCp)eff is the volumetric heat capacity (J/m3.K), keff is the effective thermal conductivity of the rock matrix (W/m.K), ρwCpw is the volumetric heat capacity of water (J/m3.K).

### Model Validation With Existing Data Set From The Literature

The numerical model was validated against the results from an existing literature study by Panja et al. (2019). They studied the feasibility of injecting hot fluid in a sedimentary basin to numerically assess the thermal performance of a hypothetical heat storage system and performed a parametric study to assess the effect of different parameters such as porosity, permeability, reservoir thickness, etc., on the thermal processes. The model consisted of three layers as a highly permeable reservoir (κx= κy =9.8x10-14 m2, κz =9.8x10-15 m2, and n=0.15) surrounded by two insulating layers (caprock and bedrock) with low permeability (κ=9.8x10-20 m2 and n=0.025) as shown in Figure 46. The initial hydrostatic pressure was 12 MPa and initial temperature was 120℃.

Figure 46 - Geometry of the reservoir simulated in Panja et al. (2019)

The physical, thermal, and hydraulic parameters used in the simulations are summarized in Table 9. These values are representative of a sedimentary formation in Canada.

Table 9 - thermal and hydraulic parameters used in the simulation.

| Formation | Thickness | Porosity | Density | Permeability | Thermal Conductivity | Rock Specific Heat |
| --- | --- | --- | --- | --- | --- | --- |
|  | (m) | (m3/m3) | (kg/m3) | (m2) | (W/m.K) | (J/kg.K) |
| Caprock/ Bedrock | 70 | 0.025 | 2500 | 9.8x10-20 | 1.05 | 770 |
| Reservoir | 110 | 0.25 | 2000 | κx= κy =9.8x10-14 κz =9.8x10-15 | 2.5 | 930 |

The operational scheme consisted of daily hot water injection for 8 hours at 250 ℃ at a rate of 40 kg/s that was followed by an extraction period of 10 hours at a rate of 32 kg/s. After extraction, the well was shut in for 6 hours. This cycle was repeated daily for 100 days. To validate the numerical model developed for this study, the initial and boundary conditions in addition to the material properties were incorporated and the system of equations were solved full coupled using a cross-platform finite element analysis, solver and Multiphysics simulation software (COMSOL Multiphysics). The temperatures and pressures from numerical simulations in this study are compared against the results from numerical simulations in Panja et al. (2019) and they are shown in Figure 16(a) and 16(b), respectively. As shown in Figures 16(a) and 16(b) there is a good comparison between the results. However, a discrepancy exists, and this is attributed to the coarser mesh used in Panja et al. (2019). Regardless, similar trends in temperature and pressure profiles are an indication of good numerical model performance.

| (a) | (b) |
| --- | --- |

Figure 47 - Numerical simulations results at the end of injection (a) Temperature (b) Pressure

### Model Calibration With Field Test

The results from the field test were used to calibrate the numerical model. Initial temperature profile given in Figure 9 was used as thermal initial condition. For the hydraulic boundary conditions, a hydrostatic pressure is applied to the domain. Then, the heat transfer between the injected fluid and the formation was calculated using temperature recorded by the DTS system and was then used as a thermal boundary condition. Losses to the surrounding formations as well as brine mixing with the injected fluid were expected. However, we were not able to incorporate these in the modeling because they were very difficult to measure in the field. The heat flux between the injected fluid and the formation is calculated using Equation 8 (Cheng, 2014) as follows:

| dQdz=2πrhw∆T | (8) |
| --- | --- |

where dQdz is the heat transfer (W/m), r is the radius (m), hw is the heat convection coefficient (W/m2.K), and ∆T is the temperature difference (K). Then, the heat convection coefficient hw is calculated based on the forced convection heat transfer mechanism, which occurs when the fluid is forced to flow due to an external source. This coefficient depends on the fluid properties, the wall surface roughness, and the type of flow (laminar or turbulent). This mechanism assumes no slip condition, i.e. that the fluid velocity at the wall surface is considered zero. This was not the case in the field test because the fluid was free to flow into the pores of the surrounding formation. However, this was not considered in the calculation and no slip condition was assumed to be valid. Then, Nusslet number (Nu) which represents the ratio of convective to conductive heat transfer can be calculated using Equation 9 as follows:

| Nu=hwDk(T) | (9) |
| --- | --- |

where D is the well diameter (m) and k(T) is the injected fluid thermal conductivity (W/(m2.K). In addition, for a flow in and across a cylinder, the Nusslet number Nu can also be calculated from Churchill and Bernstein correlation using Equation 10 as follows:

| Nu=0.3+0.62Re1/2 Pr(T)1/31+0.4Pr(T)2/31/41+Re2820005/84/5 | (10) |
| --- | --- |

where Re is the Reynolds number and Pr(T) is the Prandtl number. This correlation is valid for 3.5<Re<8000 and 0.7<Pr<380. Re is calculated using Equation 11 as follows:

| Re=ρ(T)uDμ(T) | (11) |
| --- | --- |

where ρ(T) is the injected fluid density(kg/m3), μ(T) is the injected fluid dynamic viscosity (Pa.s), and u is the fluid velocity (m/s) calculated using Equation 12 as follows:

| u=QAρ(T) | (12) |
| --- | --- |

where Q is the mass flow rate (kg/s) and A is the cross-sectional area (m2). 𝜌, 𝜇, k and Pr are temperature dependent and were varied as the temperature changed along the depth. From equation (9) and (10), the heat convection coefficient hw can be computed from which heat transfer is calculated and assigned as a heat flux boundary condition.

Figure 48 - Comparison between field test and numerical results

Comparison between experimental and numerical results are shown in Figure 48. The trend of the curve from the simulation matched that of the field results. We used this model to further investigate the operational schemes for an AGES system based on the Illinois subsurface profile and thermo-hydraulic properties of the different geological formations.

After model calibration against field tests, a 2D axisymmetric model was developed as shown in Figure 49 with coupled thermal and hydraulic processes. The vertical direction represents the different subsurface formations. In the radial direction, the cross-section is 500m, enough to limit any boundary effect errors. Initially, a single push/pull well is simulated to perform preliminary analysis. The well has a diameter of 0.34m and extends along the vertical length (z=1228m).

Cypress, target formation

Figure 49 - 2D axisymmetric model with well formations

Table 9 summarizes the physical as well as the corresponding material properties of the formations used for the analysis. The data was obtained from the existing logs provided by our project partner, Podolsky Oil Company as well as the calibrated model.

Table 9 - Thermal and hydraulic properties.

| Formation | Depth (GL) | Thickness | Porosity | Density | Permeability | Thermal Conductivity | Rock Specific Heat |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (m) | (m) | (m3/m3) | (kg/m3) | (m2) | (W/(m.K)) | (J/(kg.K) |
| Herrin Coal | 307 | 28 | 0.20 | 2569 | 3.51E-14 | 3.10 | 1320 |
| Springfield Coal | 335 | 309 | 0.17 | 2526 | 2.89E-14 | 3.63 | 1320 |
| Golconda | 644 | 87 | 0.16 | 2538 | 6.65E-15 | 3.45 | 910 |
| Menard | 731 | 38 | 0.03 | 2703 | 1.26E-17 | 3.56 | 910 |
| Vienna | 769 | 36 | 0.06 | 2653 | 4.04E-16 | 3.35 | 910 |
| Glen Dean | 805 | 5 | 0.13 | 2616 | 3.06E-16 | 2.97 | 910 |
| Tar Springs | 810 | 53 | 0.15 | 2542 | 1.22E-15 | 3.17 | 930 |
| Barlow | 863 | 5 | 0.05 | 2704 | 9.87E-18 | 3.54 | 910 |
| Cypress | 868 | 45 | 0.18 | 2681 | 2.70E-13 | 3.50 | 930 |
| Paint Creek | 913 | 8 | 0.05 | 2683 | 1.09E-17 | 3.77 | 830 |
| Benoist | 921 | 11 | 0.03 | 2686 | 1.15E-17 | 3.62 | 930 |
| Renault | 932 | 24 | 0.03 | 2692 | 2.83E-17 | 3.64 | 930 |
| Aux Vases | 956 | 17 | 0.02 | 2706 | 3.05E-17 | 3.61 | 930 |
| St Genevieve | 973 | 15 | 0.04 | 2712 | 7.84E-16 | 4.16 | 910 |
| McClosky | 988 | 27 | 0.10 | 2569 | 7.06E-14 | 4.89 | 910 |
| St Louis | 1015 | 86 | 0.13 | 2510 | 3.73E-14 | 5.14 | 910 |
| Salem | 1101 | 127 | 0.07 | 2677 | 2.71E-15 | 4.96 | 910 |

### Initial and Boundary Conditions

For the thermal boundary conditions, a fixed temperature of 12℃ on the surface was assigned. Throughout the domain, the initial temperature varied based on the geothermal gradient as shown in Figure 50(a). The geothermal gradient (24.8℃/km) was calculated from the DTS data recorded as shown in Figure 50(b). For the hydraulic boundary conditions, a hydrostatic pressure was applied to the domain with a pressure gradient of 10,000 Pa/m to represent groundwater as shown in Figure 50(b). During injection, a temperature and mass flux boundary conditions are assigned on Cypress formation as shown in Figure 50(a) and Figure 50(b).

| T=Tinlet (a) | ∂u(z,t)=No (b) |
| --- | --- |

Figure 50 - Initial boundary condition (a) Thermal (b) Hydraulic

### Simulation and Analyses

As a preliminary analysis, a single push/pull well configuration is considered. Initially, a preheating phase was performed to charge the formation for 90 days. It consisted of daily injection of hot water for 10 hours at 250 ℃ at a rate of 40 kg/s. The evolution of the thermal plume during heating is shown in Figure 51 for different days.

| Day 1 | Day 5 | Day 10 |
| --- | --- | --- |
| (a) | (b) | (c) |
| Day 15 | Day 20 | Day 30 |
| (d) | (e) | (f) |
| Day 40 | Day 50 | Day 60 |
| (g) | (h) | (i) |
| Day 70 | Day 80 | Day 90 |
| (j) | (k) | (l) |

Figure 51 - Thermal plume evolution at different cycles during charging period (a) 1 day (b) 5 days (c) 10 days (d) 15 days (e) 20 days (f) 30 days (g) 40 days (h) 50 days (i) 60 days (j) 70 days (k) 80 days (l) 90 days

After preheating the reservoir, four scenarios were performed for an analysis period of 5 years to assess the performance and efficiency of the reservoir. Scheme 1 was consistent with the approach followed by Zheng (2014). It consists of an injection period of one month at a temperature of 250 ℃ and a mass flow rate of 10 kg/s followed by a production period of one month at a mass flow rate of 10 kg/s. (2) Scheme 2 was like Scheme 1. The only parameter that varied was the mass flow rate. It was increased to 40 kg/s to study its effect. On the other hand, Scheme 3 followed the approach by Mcling et al. (2019) where each cycle included an injection of hot water at a temperature of 250 ℃ and a mass flow rate of 10 kg/s for three months to represent summer when there is excess of hot water. This period was followed by a three-month rest period in fall where no operation was performed. After that, three months of production at a mass flow rate of 10 kg/s was performed to simulate winter when demand is high. The cycle ends by three months rest period during spring. As for Scheme 4, the same cycle adopted in scheme 3 was followed. The only difference was the mass flow rate that was increased to 40 kg/s.

### Simulation Results

The temperature variation over time during injection and production was plotted for the 4 schemes simulated over the analysis period of 5 years as shown in Figure 52 (a) through (d). For all schemes, during the first few cycles, the temperature variation between the injected and produced temperature was low because the reservoir was preheated for 90 days. This variation increases as the operation continues then reaches steady state with production temperature dropping from 250oC to 207oC, 182oC, 178oC, and 164oC for schemes 1, 2, 3, and 4 respectively after a few initial cycles. In addition, the difference between the injected and produced temperature increased as the mass flow rate increased. In fact, a larger volume was being produced at higher injection rates leading to the bigger gap. When a rest period was included, a further drop in temperature at the well surface was observed after the production period. This was due to thermal dissipation inside the reservoir due to convection.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 52 - Temperature variation over time for an analysis period of 5 years (a) Scheme 1 (b) Scheme 2 (c) Scheme 3 (d) Scheme 4

The temperature profiles at mid-height of the reservoir along the radial path for the four schemes at the end of injection and production for different cycles are shown in Figure 53 to Figure 56. At the end of the charging period, the thermal plume reached ~40 m as shown on Figure 20(l). Consequently, as observed in Figure 53, Figure 54, Figure 55, and Figure 56 the temperature in the radial distance for the 4 schemes at the end of injection was highest for the first cycle. As the operation proceeded, the thermal influence zone decreased and reaches a steady state where it slightly changed from one cycle to another. Furthermore, during production, for the first few cycles, fluctuation in the thermal plume was observed. The steady state heat transfer was reached after 20 cycles and the thermally influenced zone stabilized.

| (a) | (b) |
| --- | --- |

Figure 53 - Horizontal temperature profiles at mid-height of the reservoir from Scheme 1 (a) Injection (b) Extraction

| (a) | (b) |
| --- | --- |

Figure 54 - Horizontal temperature profiles at mid-height of the reservoir from Scheme 2: (a) Injection (b) Extraction

| (a) | (b) |
| --- | --- |

Figure 55 - Horizontal temperature profiles at mid-height of the reservoir from Scheme 3: (a) Injection (b) Extraction

| (a) | (b) |
| --- | --- |

Figure 56 Horizontal temperature profiles at mid-height of the reservoir from Scheme 4: (a) Injection (b) Extraction

### Power Generation Calculations

For the single well push-pull configuration, the thermal heat production rate Wh was calculated using Equation 13 as follows:

| Wh=q×hprod | (13) |
| --- | --- |

where  q was the mass flow rate and hprod was the specific enthalpy of the fluid produced (J/kg) which varies with the temperature produced (Tprod). The heat loss Wl was then calculated using Equation 14 as follows:

| Wl=q×(hinj-hprod) | (14) |
| --- | --- |

where hinj was the specific enthalpy of the fluid injected (J/kg). Then, to estimate the electric potential of this configuration, exergy analysis was performed which assesses the maximum energy that can be produced as equilibrium was reached with a reference environment (Terzi, 2018). The efficiency f of converting the thermal energy into mechanical work was calculated using Equation 15 as follows:

| f=1-(TrejTprod) | (15) |
| --- | --- |

where Trej was the heat rejection temperature (K) (Trej used is 285.15 K (12℃) equivalent to the average annual temperature in Illinois). In equation (15), Trej and Tprod should be used in absolute temperature. The maximum mechanical work was then transformed into electric power We using a utilization efficiency of 0.45. Thus, the electric power was calculated using Equation 16 as follows:

| We=0.45fWh | (16) |
| --- | --- |

To evaluate energy recovery in the reservoir, the thermal storage efficiency ηs, which represents the ratio of the total amount of the thermal energy recovered from the reservoir to the total amount of thermal energy injected into the reservoir over a certain period, was calculated using Equation 17 as follows:

| ηs=MprodMinj | (17) |
| --- | --- |

where Minj and Mprod were the total thermal energy injected and produced into and from the reservoir during the cycle operation (unitless) respectively. They depend on the mass flow rates and the temperature of the fluid injected and produced. Minj was calculated using Equation 18 as follows:

| Minj=t1t2qinjhinjdt | (18) |
| --- | --- |

where hinj was the enthalpy of the injected fluid (J/kg) which is 1.085×106J/kg for the 4 schemes simulated (enthalpy of fresh water at a temperature of 250oC). t1 and t2 are the start and end time of the injection period. Similarly, Mprod was calculated using Equation 19 as follows:

| Mprod=t1t2qprodhproddt | (19) |
| --- | --- |

Changes of heat production  Wh, heat loss Wl, electric power We, and thermal storage efficiency respectively ηs over the 5 years analysis period for the different schemes are shown in Figure 57(a) through Figure 57(d). The heat production Wh, electric power We, and thermal storage efficiency respectively ηs depend on the mass flow rate and the temperature produced. The mass flow rate was constant over the analysis period (5 years) while the temperature period was varying every cycle. Thus, the three curves followed the same trend as the temperature produced for the four schemes. Initially, the temperature dropped which was translated into a decrease in the heat production Wh, electric power We, and thermal storage efficiency respectively ηs curves. Then, after multiple cycles, the temperature produced reached a steady state value which was also observed on the curves. On the other hand, the heat loss Wl increased at the beginning then reached a constant value. This was also consistent with the temperature profiles in Figure 57 where the difference between the injected and produced temperature started low but increased after multiple cycles to reach a stable value. Furthermore, as the injection and production period increased, the efficiency tended to decrease. This was mainly because as the fluid was extracted for a longer period, the temperature produced decreased along with the thermal plume. In addition, mass flow rate also affected the results. For lower mass flow rates, the efficiency was the highest and the heat loss was the lowest among the 4 schemes simulated.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 57 - Evolution of (a) heat production  Wh (b) heat loss Wl (c) electric power We  (d) thermal storage efficiency respectively ηs with time

The average electric power We and the corresponding generated power over a period of 5 years for the different schemes proposed are summarized in Table 10. The first scheme results in the highest power generation with 325.74 GWh, followed by scheme 3 (251.23 GWh), then scheme 2 (251.23 GWh), and lastly scheme 4 (205.68 GWh). This was expected since the efficiency of the first scheme was the highest.

Table 10 - Average electric power and power generation over 5 years

|  | Average Electric Power (MW) | Power Generation (GWh) | Description |
| --- | --- | --- | --- |
| Scheme 1 | 7.44 | 325.74 | One Month Injection at 250 ℃-10 kg/s. One Month production at 10 kg/s. |
| Scheme 2 | 5.74 | 251.23 | One Month Injection at 250 ℃-40 kg/s. One Month production at 40 kg/s. |
| Scheme 3 | 6.26 | 274.22 | Three Month Injection at 250 ℃-10 kg/s. Three Month production at 10 kg/s. (Seasonal rest periods) |
| Scheme 4 | 4.70 | 205.68 | Three Month Injection at 250 ℃-40 kg/s. Three Month production at 40 kg/s. (Seasonal rest periods) |

# Phase I Market Research & Business Model

The initial market research was conducted and supplemented with participation in the Energy I-Corps for SBIR / STTR program for customer discovery in the Fall of 2020, the business model was refined after interviewing industry SMEs. Interviewees were from solar, lithium-ion, utility, storage farm, and power production industries—all of which helped provide parameters to determine investment interest and the market demands for this technology. The original business model was limited to capturing heat through setting AGES with Concentrating Solar-thermal Power (CSP) technology (for instance parabolic trough collector). Several iterations of the business model were developed throughout the customer discovery process. While several target applications were identified through this process, nearly all interviewees advised that arbitraging wholesale electricity prices would provide the greatest benefit with roads to expand into the other areas after the technology is proven via arbitrage. The primary industry demand for long duration energy storage was overwhelmingly apparent from the industry experts. Hence, the customer discovery process led to the conclusion that the electricity arbitrage model may be the most viable. For the purposes of the commercialization plan we elected to use this application to develop a business model. However, Projeo maintains a position that is open to all target applications and believes the best path towards commercialization will be through demonstrating commercial viability by developing the first few projects. The key to success of the adoption of this process will lie in the development of a demonstration project to show commercial success (whatever the target application). A high-level of support for this project within the oil and gas industry and the energy and utility industry was observed during the customer discovery market research and is supported by the letters of support attached to the commercialization proposal.

The economic analysis conducted in Phase I was built on an electricity arbitrage model. Further, the team will conduct additional customer discovery during future research, in addition to the customer market research undertaken by LARTA, to expand the economic model for other applications including AGES industrial applications, direct heating applications, loss of utility and grid resilience applications. The team will evaluate the economic models and pivot to the appropriate application based on the future research results and market demand.

### Economic Model

An AGES system allows for long-duration energy storage with many potential applications of the technology. The simplest application is to utilize the longer-duration storage to arbitrage electricity by purchasing power in off-peak hours, storing, and reselling the electricity during peak times. A standard method of assessing project finances is to determine the net present value (NPV) or the project. Net present value is defined as the following:

| NPV=Σ Cash Flow, Year n1+rn, where r=discount rate and n=project year | (20) |
| --- | --- |

Using an assumed 40 MWe capacity for a 10 well system to purchase, convert (via electric boiler), store geothermally, and discharge for sale via steam power block, and a discount rate of 15%, the project NPV, per the DOE provided worksheet, is $9,715,000 with a DOE investment Multiple of 8.4.  The project is expected to break even during year 6 of commercial operations, post commercial expansion (doubling of project size). An additional metric used to determine the financial and economic viability of an arbitrage AGES venture is the operating margins of the project. Operating margin is defined as:

| Operating Margin= Operating IncomeRevenue (Sales) | (21) |
| --- | --- |

Operating margins are estimated to be extremely high, all over 50% in years without large capital expenditures. Net cash flows are positive in every year without large capital expenditures, which will occur in commercial years 1 and 4, projected to be in 2023 and 2026.  The revenue stream outlined in the commercialization plan reflect the purchase of electricity in off-peak hours at price lows, and the revenue from sale of the electricity during peak-demand hours on the MISO grid. Revenues are expected to begin in the first year of commercial operations, with profit occurring in year 2, and in every subsequent year without capital expenditures in project expansion. The assumptions for the profit reflect an anticipated 90% storage efficiency (Wendt, 2019), and 40 hours of electric purchase and 40 hours of electric sales, on average, per week each year. Additionally, expenses are projected to rise at 1% per year, as are electricity prices—as both a purchaser and seller. Based on our outputs from the model, we have determined our LCOE to be 13.8¢ per kWh. LCOE is a standard metric to determine the baseline cost of electricity generation at a $/kWh, which is defined as:

| LCOE= NPV, Project Costs ($)NPV, Electricty Output (kWh) | (22) |
| --- | --- |

The determined LCOE of the AGES operations is $0.138/kWh. Other AGES modeling has determined the LCOE of projects utilizing on-site solar for electricity generation at $0.1307/kWh but have a higher capital cost to begin operations and does not include an electricity purchase price as our operating costs require due to the arbitrage business model (Wendt, 2019). With a high operating margin, the project is extremely economically viable and profitable. Due to our continued research regarding the economics and revenues surrounding the AGES technology in Phase II, we felt the continued economic research was imperative to later developing the Monte Carlo model. We used resources instead to further our market research for the technology, establishing business connections in related industries, and the refinement of the Phase I Field Test.

### Team for Phase II

The team has a strong oil and gas, permitting, sales, commercialization, simulation, technical, and research background. As part of the Phase I customer discovery effort and Phase II commercialization plan development, certain knowledge gaps were identified. The team has included SMEs on geothermal infrastructure, grid integration, tax incentives and policy regulations, introduced below.

Mr. Kevin Kitz has been included in the Phase II proposal as a consultant to guide and advise the team on technical aspects of geothermal surface infrastructure, and AGES parameters. Mr. Kitz was the key advisor for the power optimization process for the Idaho National Lab’s Geological Thermal Energy Storage project. Mr. Kitz has designed and operated many different types of geothermal power plants and holds patents on geothermal steam purification, geothermal fluid treatments and prevention of stainless cracking in geothermal brine. As part of the customer discovery and market research during Phase I, several metrics evaluated by target customers were identified. Mr. Kitz has the experience to advise on the development of the metrics required by the target customers. Mr. Kitz has also worked with utility companies to evaluate the potential of AGES on their system and can assist the team similar discussions with utility companies.

Mr. Warren Lavey is an adjunct professor at the University of Illinois at Urbana-Champaign.  After earning graduate degrees in law, applied mathematics and economics at Harvard and Cambridge Universities, he worked on technology and environmental policies, laws, and transactions in both the federal government and private sector.  He published articles, taught courses, and testified on incentives and regulations applicable to renewable energy and energy storage systems. Mr. Lavey has been included in the team as a consultant to provide guidance energy storage policy and tax incentives. Designing and deploying the energy storage system will depend in part on the availability of government tax and other incentives as well as energy grid and utility regulations.  Additionally, laws pertaining to uses of abandoned hydrocarbon wells will affect the design, location, and costs of the proposed energy storage systems.  Mr. Lavey will assist the team assess applicable legal frameworks, which vary across states and are subject to changes via federal and state legislation and regulations. By integrating legal and policy expertise at this stage in the technology development, the team will be able to consider critical factors in guiding research and making decisions which will influence the success of the energy storage system.

In addition, CMY Solutions a grid integration evaluation company has been engaged as a consultant. They have conducted several studies for alternative generation sources including wind and solar with multiple utilities and are experienced in creating and interpreting hosting capacity maps using existing utility models. CMY solutions will assist the team with grid integration studies and evaluate potential sites with utility hosting capacities.

In addition, we have included Petrolern, as a consultant. Petrolern is actively involved in geothermal energy development from play fairway mapping and feasibility studies to design and modeling of enhanced geothermal systems in addition to repurposing late-stage oil and gas wells to geothermal sources of heat and electricity. Projeo’s expertise in O&G project management and operational expertise would work in synergy with the GIS spatial analysis and data mining services offered by Petrolern LLC. The organization chart in Figure 58 summarizes the consultants and partners involved in the phase II proposal.

Figure 58 - Organizational Chart of the Project team (SBC, Research Institution, Consultants and Partners).

### Additional Custom market Research

To build on the current market research conducted by Projeo, LARTA organization was engaged to acquire a custom market research. The custom market research was focused on applications for the AGES technology beyond arbitraging wholesale electricity, in industrial residential, loss of utility or direct use heating applications. A market size estimate and market ecosystem were developed in the custom market research. Additionally, a list of contact for potential customer, partners and investors was also developed in the custom market report.

# Lessons Learned

The lessons learnt during phase I are summarized below:

- The procedures applied in converting the oil producing well into a thermal storage well provide an initial standard for rehabilitation of O&G wells for thermal energy storage application. Although, each well requires its own evaluation to identify the rehabilitation approaches to be pursued, the field procedure applied serves as a standard.

- Heat losses in the well were higher than expected and due to limited budget on the phase I, we were not able to implement the preventative measures to reduce heat losses. However, several modifications during the experiment were successful to reduce the heat losses (injection rate/travel time, wellbore volume etc.)

- The target reservoir selected satisfied the thermal and petrophysical requirements for the proposed field test. However, selecting a shallower formation would reduce the well bore heat losses and reduce the operational costs associate with managing heat losses.

- The availability of petrophysical and geologic data was evaluated during the site selection process. This data was helpful in evaluating the injectivity (porosity/permeability) of the selected target reservoir and saved operational costs of stimulating the reservoir rock.

- The customer discovery effort guided by the Energy I-Corps program was very helpful in early pivots to market strategy. Several iterations to the market strategy were developed as the market ecosystem understanding was refined through the process. Early customer discovery efforts are crucial in guiding the research to acquire the data required for developing a viable product for the target customer and market.

- The site selected for the field test in phase I, was located on a lease land without municipal water or power supply setup. The cost of operations can be minimized drastically by selecting a site with easier access to water and energy.

- The site lease land was evaluated for accessibility, but due to severe wet weather conditions roads were in accessible or needed repairs. This caused operational delays and additional cost in site road maintenance. To evaluate the effects of unexpected weather and setup contingent site-hosts have been identified for future field tests.

# Conclusion

The Phase I project research was built around a strong target reservoir evaluation, based on available petrophysical, and geologic data in the Illinois basin. Additionally, a productive initial customer discovery was performed with several use cases identified for the AGES technology and the electric arbitrage case receiving cost analysis treatment. The initial modelling and simulations provided the basis of designing a field test to calibrate and validate the built models. Similarly, the customer discovery process also identified the parameters essential for the AGES technology to be profitable.

The Team was successful in accomplishing the objectives outlined for the Phase I project research. The regulatory process of converting orphaned or abandoned wells into an asset for AGES application were identified and initial steps towards establishing a procedure were also implemented. The team was able to pick a reservoir that was prevalent across the Illinois basin for the purpose of profiling a potential reservoir for a commercial project. Preliminary analysis of the data acquired from the field test validates the hydrogeological characteristics of subsurface reservoirs in the Illinois Basin needed for heat storage. The data acquired was used to calibrate and improve the modelling and simulations, used in identifying the target reservoir in the Illinois Basin. Several lessons learnt were recognized that will assist in refining the processes developed in the phase I research to conduct future research more efficiently. The processes developed are an initial attempt to providing a standard for a commercial scale project converting oil and gas infrastructure for flexible electricity generation.

The market research conducted assisted in developing an outline of a commercial scale AGES project, which is supported by industry professionals in both oil and gas and the utility and energy industry. Additionally, several parameters evaluated by the target customers were identified and SMEs proficient in geothermal infrastructure, grid integration, tax incentives and policy regulations were added to the team in the future research. The exceedingly qualified team assembled for the future research projects will be able to develop the initial phase I research and acquire metrics required to present this project to venture capital firms providing private investment to fund long duration energy storage solutions to developing a long duration subsurface energy storage solution to accelerate renewable energy penetration across the nation, provide solutions for improved grid resilience, support the national energy resilience objectives, and serve to stabilize volatility in energy supply-demand and energy prices.

The phase I project was successful. The outcome of the Phase 1 project indicated that a larger scale test is worth investment and required to further evaluate the technical feasibility, develop the commercial and economic models, and calibrate those models to provide the key metrics used by potential investor to invest in a commercial scale AGES project.

# References

Cheng, W. L., Li, T. T., Nian, Y. L., & Xie, K. (2014). An analysis of insulation of abandoned oil wells reused for geothermal power generation. Energy Procedia, 61, 607-610.

CAT PUMPS. Data Sheet Brass Model 230. n.d.

Cat Pumps. Data Sheet Brass Model 740. n.d.

Mcling, T., Wendt, D. S., Dobson, P., Doughty, C., Spycher, N., Roberson, D., & McLaughlin, F. H. (2019). Dynamic Earth Energy Storage: Terawatt-Year, Grid Scale Energy Storage Using Planet Earth as a Thermal Battery (AGES): Final Seedling Project Report. Idaho National Lab.(INL), Idaho Falls, ID (United States).

Panja, P., McLennan, J., & Green, S. (2020, September). Temperature and Pressure Profiles for Geothermal Battery Energy Storage in Sedimentary Basins. In 54th US Rock Mechanics/Geomechanics Symposium. American Rock Mechanics Association.

Richard C. Berg. n.d. “Illinois State Geological Survey Nonconventional CO2-EOR Project Description | ISGS.” Retrieved May 10, 2021 (https://isgs.illinois.edu/research/erd/nco2eor/project-description).

Terzi, R. (2018). Application of exergy analysis to energy systems. Application of Exergy.

Zheng, Q. (2014). Design of a high temperature subsurface thermal energy storage system.

# Appendices

### Appendix A: Field Activity Report.

### Appendix B: Field Program.

# Feasibility of Storing Heat in the Subsurface For Flexible Electricity Heat Storage

# M. Eckleberry #4 Field Testing Program

Geothermal Field Test Procedure

# Well Information

Name:		M. Eckleberry #4

Lease:		Mary Eckleberry – West Jeff Unit

Drilled: 	8/19/2013 – 8/22/2013

Permit #:	063355

Location:	480' North and 660' West of the Southeast Corner Northeast Quarter of the Northeast Quarter of Section 16, Township 01S Range 07E of Wayne County, IL.

Lat 38.440375 Lon -88. 424731

Elevation: 	446’ GL & 5’ KB

# Well Details

Well Sizes:

| Bit Size | Casing Size | Depth (Hole/Csg) | Weight/Grade | Burst/Collapse |
| --- | --- | --- | --- | --- |
| 12-1/4” | 8-5/8” | 263’ / 253’ | Unknown | Unkown |
| 7-7/8” | 4-1/2” | 3,146’ / 3,126’ * | 10.5/J-55** | 3592psi/3007psi** |

*	Production Open hole (3126’ – 3146) – Aux Vases
**	Burst / Collapse calculated from assumed weight and grade with 25% S.F.

TOC:	Surface casing circulated to surface
Production casing estimated at 1,300’. 
Lead 200 sks of 65/35 Poz-A blend w/6% gel and FLA
Tail: 150 sks of 50/50 Poz-A blend w/ 3% KCL and 2% Calseal

Condition: 	Sucker rod pump, and tubing has been retrieved.

# Heat Storage Formations

Possible Targets (Per CEE Simulation):

- Cypress Middle (Top 2,905’ or 2,902’)

- Barlow (Top 2,833’ or 2,828’)

- Benoist (Top 3,023’ / 3,016’)

Table 1- Formation Tops

| Formation Name | Formation Top Depth (ft) |
| --- | --- |
| (Label) |  |
| Bethel | 3,011 |
| Paint Creek Lower | 3,021 |
| Benoist | 3,016 |
| Renault | 3,057 |
| Renault Upper | 3,064 |
| Renault Lower | 3,122 |
| Aux Vases | 3,136 |
| St Genevieve | 3,193 |
| Fredonia | 3,205 |
| Ohara | 3,206 |
| Ohara Lower | 3,210 |
| McClosky | 3,242 |
| St Louis | 3,331 |
| Salem | 3,612 |
| Warsaw | 4,030 |

| Formation Name | Formation Top Depth (ft) |
| --- | --- |
| (Label) |  |
| Herrin Coal No 6 | 1,008 |
| Springfield Coal | 1,099 |
| Golconda | 2,114 |
| Menard | 2,398 |
| Menard Lower | 2,448 |
| Vienna | 2,523 |
| Glen Dean | 2,642 |
| Glen Dean Lower | 2,651 |
| Tar Springs | 2,658 |
| Barlow | 2,828 |
| Cypress Upper | 2,847 |
| Cypress | 2,872 |
| Cypress Lower | 2,892 |
| Cypress Middle | 2,902 |
| Paint Creek | 2,996 |

# Field Program: Heat Injection

WEEK 1 (3/22/21 – 3/28/21)

Prep Work

- Mobilize frac tank

- Fill frac tank with water

- Deliver wellhead Tee and fittings.

- Deliver Generator

- Day 1 – Mobilize wireline:

  - Gauge ring run – 2 hr

  - Spot CIBP at 3,100’ and dump bail cement – 3 hr

  - Load the hole with fresh water and RIH with CCL+CBL+GR, packoff – 3 hr

  - Pressure pass with wireline PCE if necessary.

  - Pressure test the plug and the well casing - 1 hr

  - Simultaneously:

    - Install pressure gauges on well head casing to monitor remotely.

    - Setup the generator.

    - Setup 2” trash pump, pressure washer pump, 2“suction hoses, discharge hoses, high pressure ½” hoses and return pit.

  - Finalize perf interval overnight based on GR log.

- Day 2 – Mobilize swab rig:

  - Rig-up swab unit and open top tank, Install 4-1/2” valve and Tee– 2 hr

  - Casing swab well before perforating – 4 hr

    - Check if plug is holding by fluid level.

    - Swab below planned perforation depth.

  - Get an echo meter shot of fluid level. – 0.5 hr

  - RIH with packoff, perforating gun, tag cement, and correlate. – 0.5 hr

  - Packoff, shoot echometer, perforate Cypress (~2,958’ – 2,963’). Wait ~20 minutes to see initial buildup (echometer and pressure gauge). POOH wireline. – 2 hr

  - Shut-In Well and monitor surface buildup overnight.

- Day 3 – Additional swabbing and sampling:

  - Shoot echometer for overnight production – 0.5 hr

  - Bleed down well. – 0.5 hr

  - If fluid gain is less than 1000 ft, discuss acid job or additional perfs and skip sampling.

  - Swab to ~500’ of perforations. Catch and retain samples on last 2 swab pulls. – 2 hr

  - RIH with Kuster sampler/pressure gauge on sand line, get a fluid sample – 2 hr

    - Bring samples back to ISGS. ISGS to provide analysis (minimum of pH and density)

  - Release the rig.

  - Complete Rig-up simultaneously (heater, propane, instrumentation).

- Day 4 –Instrumentation Setup

  - Setup CAT pump

- Wire Murphy Shutdowns

- Wire electric for CAT pump

- Spot Heater with Propane and Flow Lines

- Instrumentation

    - Surface Casing Pressure - 5,000 PSI Digital and Analog

    - Surface Flow Rate – 0-7 GPM during Long Term Test, during high rate injection, Ultrasonic as backup for both

    - RTD Temperature – In flow line after heater

    - Housing or location for DTS interrogator (trailer)

    - Data Logger / PC

    - Hotspot

    - Solar or other power for pressure and DTS interrogator post-injection

    - Site Camera

    - Fill Propane

- Deliver DTS cable mounted on spooler to wellsite.

- PRESSURE TEST SURFACE RIGUP.

- Day 5 – Extra Day (Instrumentation Setup)

- Day 6-7 – SD for Weekend

WEEK 2 (3/29/21 – 04/04/21)

- Day 8 – Install DTS

  - Perform DH splice and install memory Gauges – 3 hr

  - RIH with DTS and P/T Memory Gauge to ~3,100’ – 4 hr

  - Land DTS in Well Head – 2 hr

  - Setup DTS Interrogator – 2 hr

- Day 9 – Extra Day (DTS Setup), Pressure Test Heater, etc.

- Day 10 – Well Testing:

  - Acquire freshwater sample for analysis (from frac tank).

  - Perform a step rate test and a short-term injection test, shut down for Pressure fall off. – 8 hr

- Day 11-14 – PFO / SD for Weekend

WEEK 3 (04/05/21 – 04/11/21)

- Day 15-21 - Inject hot water for one week (24/7).

WEEKS 4-5 (04/12/21 -04/25/21)

- Day 22-35 - Shut in and monitor temp decay 2-3 weeks.

WEEK 6 (04/26/21 – 05/02/21)

- Day 36-37 – Flow back to frac tank until pressure equalizes.

- Day 38 – Retrieve DTS from well, download memory gauges, rig down field equipment.

- Day 39-40 – Rig down field equipment.

- Day 41-42 – SD for weekend.

WEEK 7 (05/03/21 – 05/09/21)

- Day 43-49 – Extra Week.

WEEK 8 (05/10/21 – 05/16/21)

- Day 50 – Mobilize pulling unit. Cut and pull casing, plug well.

- Day 51 – Cut and pull casing, plug well.