Table 1. Conditions of hydraulic fracturing experiments, FY14

| Exp. No. | Core name | Temp. (°C) | Pressure (atm) | Fracking fluid | Effective pressure of rock fracking. (atm) | Permeability after fracking exp. (mD) | Gas leaking test |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Coso-1-10 | 300 | 333 | PAA-CO2 | 17 | 8.8 | Yes |
| 2 | Coso-1-19 | 300 | 333 | DIW/CO2 | >170 a | < 0.001 b | No c |
| 3 | Newberry-01 | 300 | 333 | PAA-CO2 | 114 | 0.031 | Yes |
| 4 | Newberry-03 | 300 | 333 | PAA-CO2 | 82 | 0.032 | Yes |
| 5 | Newberry-02 | 300 | 333 | DIW/CO2 | > 170 a | <0.001 | No c |
| 6 | Coso-1-9 | 150 | 250 | PAA-CO2 | 34 | 8.8 | Yes |
| 7 | Coso-1-18 | 150 | 250 | DIW/CO2 | 49 | <0.001 | Yes |
| 8 | Newberry-04 | 150 | 250 | PAA-CO2 | 133 | 5.3 | Yes |
| 9 | Newberry-08 | 150 | 250 | DIW/CO2 | >170 a | <0.001 | No |
| 10 | Coso-1-14 | 300 | 333 | PAA (pH7)-CO2 | 27 | 2.0 | Yes |
| 11 | Coso-1-16 | 300 | 333 | pH 7 buffer-CO2 | 45 | 0.60 | Yes |

a: No communication observed even at maximum pressure allowed by the pump. Also, gas leaking test and  XMT analysis did not indicate any fractures on this rock core; b: no liquid was observed on the external surface even after applying 10 atm pressure to the center hole for 5 min,  suggesting the permeability is lower than 0.001 mD. c: no gas flow observed

## Fig. 1. Schematic diagram for the hydraulic fracturing experimental setup.

Fig. 2. XMT images of Coso 1-10 before (left) and after (right) hydraulic fracturing experiment. Control experiment (#2, coso-1-19) did not show the presence of fractures even at differential pressures as high as 170 atm. The fractures extended ~0.5 mm above and ~17 mm below this transversal section.

## Fig. 3. XMT images of rock cores Coso 1-9 before (left) and after (right) hydraulic fracturing experiment. Control experiment (#8, Coso 1-18) did not show the presence of fractures. The fractures extend ~10 mm above and ~5 mm below this transversal section.

## Fig. 4. XMT images of Newberry-04 rock core fractured with PAA-CO2 fracturing fluid. The Figure shows sample micrographs before (left) and after (right) hydraulic fracturing experiment. The fractures extend ~0.4 mm above and ~1.5 mm below this transversal section.   Control experiment (Newberry-08) did not show the presence of fractures.

Fig. 5. Representative XMT images of Coso 1-14 (a, b) and Coso 1-16 (c, d) rock cores before and after hydraulic fracturing experiment. The arrows point to the fractures. Note the much larger aperture of the fractures created on Coso 1-14.

## Fig. 6. CFD model of Coso 1-10 showing: left: isosurface of fractures 1 and 2 for Coso 1-10 with XMT data (left). The inlet was extruded during volume meshing to improve solution convergence. Middle: CFD model of Coso 1-10 showing velocity vectors colored by absolute velocity. The flow rate and velocity of the flow in fracture 1 is higher than fracture 2 due to smaller cross-sectional area but higher fracture volume. Right: contours of static pressure showing gradient along the fracture wall.

## Fig. 7. CFD model of Newberry-04 showing: Left: a slice of XMT data with isosurface generated using segmented data for Newberry 4 sample. The interface between the rock and the rock core cradle was assumed as the outlet for the CFD model. Middle: CFD model of Newberry-04 showing velocity vectors colored by absolute velocity. Volumetric flow and fluid velocity in both fracture 1 and 2 are similar since the volumes and apertures of fractures 1 and 2 are comparable. Right: contours of static pressure.